# WS4 METAL WALL CLADDING | Hook-on system

| Pos ............... ............... m2 | Supply and install durlum WS4 metal wall cladding panels Panels shall be positively engaged [positive-fit] and installed free of stress into a dedicated substructure [see separate item]. Tool-free demounting is ensured. Tolerances and quality requirements in accordance with TAIM, DIN EN 13964 and durlum standard. Material: galvanized steel sheet Material thickness: 1.00 mm Finish: powder-coated, RAL colour …………… [e.g., durlum RAL 9016 matt, DUROPLAN T1] Coating thickness: approx. 60 µm Perforation: type …………… [e.g., durlum RG-L15] Acoustics: durlum acoustic fleece backing, colour black Panel configuration: rectangular; vertical orientation Hook-in profiles: factory-fitted [2 pcs up to 1800 mm; 3 pcs 1801-3000 mm]; corners factory riveted System depth: approx. 50 mm [panel + substructure]; max. recommended panel area: 2.25 m2 Dimensions: height: …………… mm [max. 3000 mm] width: …………… mm [max. 1250 mm] EP: …………… [EUR/m2] GP: …………… [EUR] |
| --- | --- |
| Pos ............... ............... pcs | Set-up / tooling costs per panel size, detailing and call-off [1-10 / 11-250 / >250 pcs per line item] EP: …………… [EUR/pcs] GP: …………… [EUR] |
| Pos ............... ............... m | Supply and install a durlum substructure for system WS4 Substructure consists of WS4 1100 wall Z-profiles securely fixed to the load-bearing substrate using approved anchors/fixings selected to meet local project requirements. Profile spacing shall be set out per installation instructions and coordinated with the spacing of the factory-fitted hook-in profiles to ensure positive engagement and stress-free installation. Install Z-profiles level and in alignment. After installation, WS0 100 decouplers shall be pushed onto the Z-profiles on site [quantity/spacing per installation instructions]. Only manufacturer-approved components may be used. EP: …………… [EUR/m] GP: …………… [EUR] |
| Pos ............... ............... m | Provisional item [if required]: Supply and install durlum base profile WS0 200 for system WS4 Function: Lower termination of the wall cladding Material: galvanized steel sheet, 1.00 mm Finish: powder-coated, RAL colour …………… [e.g., durlum RAL 9016 matt, DUROPLAN T1] Coating thickness: approx. 60 µm Profile geometry: 10/65/10/25 mm Length: 3000 mm EP: …………… [EUR/m] GP: …………… [EUR] |
| Pos ............... ............... pcs | Set-up / tooling costs base profile WS0 200 per size, detailing and call-off EP: …………… [EUR/pcs] GP: …………… [EUR] |
| Pos ............... ............... m | Provisional item [if required]: Supply and install durlum termination profile WS0 300 for system WS4 Function: Lower, upper or side termination of the wall cladding Material: galvanized steel sheet, 1.00 mm Finish: powder-coated, RAL colour …………… [e.g., durlum RAL 9016 matt, DUROPLAN T1] Coating thickness: approx. 60 µm Profile geometry: 50/50 mm Length: 3000 mm EP: …………… [EUR/m] GP: …………… [EUR] |
| Pos ............... ............... pcs | Set-up / tooling costs base profile WS0 300 per size, detailing and call-off EP: …………… [EUR/pcs] GP: …………… [EUR] |