# WS7 METAL WALL CLADDING | Carrier rail system

| Pos ............... ............... m2 | Supply and install durlum wall panels, system WS7 Wall panels shall be positively engaged [positive-fit] and installed free of stress into a dedicated carrier rail substructure [see separate item] and secured against mechanical pressure loads by WS7 panel locks [see separate item]. Tool-free demounting is ensured. Tolerances and quality requirements in accordance with TAIM, DIN EN 13964 and durlum standard. Material: galvanized steel sheet Material thickness: 1,0mm Finish: powder-coated, RAL colour …………… [e.g., durlum RAL 9016 matt, DUROPLAN T1] Coating thickness: approx. 60 µm Perforation: type ............... [e.g., durlum RG-L15] Acoustics: durlum acoustic fleece backing; colour black Panel configuration: rectangular, horizontal orientation Dimensions: length: …………… mm [max. 3000 mm] width: …………… mm [max. 1250 mm] EP: …………… [EUR/m2] GP: …………… [EUR] |
| --- | --- |
| Pos ............... ............... pcs | Set-up / tooling costs wall panel WS7 per panel size, detailing and call-off [1-10 / 11-250 / >250 pcs per line item] EP: …………… [EUR/pcs] GP: …………… [EUR] |
| Pos ............... ............... m | Supply and install a durlum substructure for system WS7 Substructure consisting of carrier rail profiles W2053-ST into which the wall panels shall be positively engaged [positive-fit] and installed free of stress. Carrier rail profiles shall be securely fixed to the load-bearing substrate using approved anchors/fixings selected to meet local project requirements. Profile spacing shall be determined based on WS7 panel dimensions and the installation instructions to ensure positive engagement and stress-free installation. Install profiles level and in alignment. Only manufacturer-approved components may be used. All load-bearing parts shall be made of galvanized steel sheet. EP: …………… [EUR/m] GP: …………… [EUR] |
| Pos ............... ............... m | Supply and install WS7 1100 wall Z-profiles as the upper and lower hanging rails for system WS7 Z-profiles shall be securely fixed to the load-bearing substrate using approved anchors/fixings selected to meet local project requirements. Profile spacing shall be determined based on WS7 panel dimensions and the installation instructions to ensure positive engagement and stress-free installation. Install profiles level and in alignment. EP: …………… [EUR/m] GP: …………… [EUR] |
| Pos ............... ............... pcs | WS7 panel lock U2053-WS7 Provide and install panel locks to secure the wall panels against mechanical pressure loads. A panel lock is required at each panel joint. EP: …………… [EUR/pcs] GP: …………… [EUR] |
| Pos ............... ............... m | Provisional item [if required]: Supply and install durlum base profile WS0 200 for system WS7 Function: Lower termination of the wall cladding Material: galvanized steel sheet; t=1,0 mm Finish: powder-coated, RAL colour …………… [e.g., durlum RAL 9016 matt, DUROPLAN T1] Profile geometry: 10/65/10/25 mm Length: 3000 mm EP: …………… [EUR/m] GP: …………… [EUR] |
| Pos ............... ............... pcs | Set-up / tooling costs base profile WS0 200 per size, detailing and call-off EP: …………… [EUR/pcs] GP: …………… [EUR] |
| Pos ............... ............... m | Provisional item [if required]: Supply and install durlum termination profile WS7 300 for system WS7 Function: Lower, upper or side termination of the wall cladding Material: galvanized steel sheet; t=1,0 mm Finish: powder-coated, RAL color …………… [e.g., durlum RAL 9016 matt, DUROPLAN T1] Profile geometry: 50/50 mm Length: 3000 mm EP: …………… [EUR/m] GP: …………… [EUR] |
| Pos ............... ............... pcs | Set-up / tooling costs termination profile WS7 300 per size, detailing and call-off EP: …………… [EUR/pcs] GP: …………… [EUR] |